# Buck Converter Modeling in Simulink:

# Switched and Averaged Models

Modeling, simulation, and validation for control-oriented design

% Author: Luis Gaona

% Date: 2026

## 1 Overview

This live script presents the modeling and simulation of a DC-DC Buck converter using both switched and averaged representations in Simulink. The averaged model is intended for control design and fast simulation, while the switched model captures high-frequency switching behavior and is used for validation of the averaged dynamics. The objective is to compare both models in terms of output voltage, inductor current, and computational efficiency.

## 2. System Description

### 2.1 Buck Converter Topology

The Buck converter consists of a controlled switch (MOSFET), a diode, an inductor, an output capacitor, and a resistive load. Energy is transferred from the input source to the load by controlling the duty cycle of the switching device.

### 2.2 Modeling Assumptions

Continuous Conduction Mode (CCM)

Ideal switch and diode

Constant input voltage

Negligible parasitics

Ideal output capacitor

## 3. Mathematical Model

### 3.1 State Variables

The system is described using the following state variables:

| Variable | Description |
| --- | --- |
|  | Inductor current [A] |
|  | Output capacitor voltage [V] |
|  | Duty cycle (0 <= D <= 1) |

### 3.2 Switched Model Equations

The switched model alternates between two linear subsystems depending on the switching state.

On state

Off state

This representation captures switching ripple and high-frequency dynamics.

### 3.3 Averaged Model

The averaged model replaces the switching behavior by its duty-cycle weighted average.

Under steady-state CCM operation:

## 4. Simulink Implementation

### 4.1 Switched Model

The switched model is implemented in Simulink using:

A PWM generator operating at fixed switching frequency

Conditional logic to select ON and OFF state equations

Fixed-step discrete solver

Model file:

thisFile = matlab.desktop.editor.getActiveFilename;

projectRoot = fileparts(thisFile);

modelsDir = fullfile(projectRoot, '..', 'models');

assert(isfolder(modelsDir), 'Models folder not found: %s', modelsDir)

open_system(fullfile(modelsDir, 'buck_switched.slx'))

### 4.2 Averaged Model Implementation

The averaged model is implemented without explicit switching. This allows larger simulation time steps and faster execution.

Model file:

open_system(fullfile(modelsDir, 'buck_averaged.slx'))

## 5. Simulation Parameters

### 5.1 Electrical Parameters

|  | Input voltage [V] |
| --- | --- |
|  | inductance [H] |
|  | Capacitance [F] |
|  | Load resistance [Ohm] |
|  | Duty cycle |

Vin = 28;          % Input voltage [V]

L   = 50e-6;      % Inductance [H]

C   = 500e-6;      % Capacitance [F]

R   = 3;          % Load resistance [Ohm]

D   = 0.536;         % Duty cycle

fprintf('Vin = %.1f V, D = %.2f\n', Vin, D)

### 5.2 Solver Configuration

Switched model:

Fixed-step, Ts = 1e-7 s

Averaged model:

Fixed-step, Ts = 1e-5 s

These configurations ensure numerical stability while balancing accuracy and simulation speed.

## 6. Simulation Results

### 6.1 Run Simulations

Run both models and collect results using To Workspace blocks.

sim_switched = sim('buck_switched.slx');

sim_averaged = sim('buck_averaged.slx');

### 6.2 Output Voltage Comparison

The averaged model should match the mean value of the switched model.

t_sw = sim_switched.vout.time;

v_sw = sim_switched.vout.signals.values;

t_av = sim_averaged.vout.time;

v_av = sim_averaged.vout.signals.values;

figure

plot(t_sw, v_sw)

hold on

plot(t_av, v_av, '--')

grid on

xlabel('Time [s]')

ylabel('Output Voltage [V]')

legend('Switched Model','Averaged Model')

title('Output Voltage Comparison')

### 6.3 Inductor Current

The switched model exhibits ripple, while the averaged model represents the mean inductor current.

t_iL = sim_switched.iL.time;

iL   = sim_switched.iL.signals.values;

figure

plot(t_iL, iL)

grid on

xlabel('Time [s]')

ylabel('Inductor Current [A]')

title('Inductor Current (Switched Model)')

## 7. Reproducibility

To reproduce these results:

Open buck_switched.slx

Open buck_averaged.slx

Run this live script

Modify duty cycle D and rerun the simulations

## 8. Limitations

Valid only under CCM operation

Switching losses are not modeled

Not suitable for EMI or thermal analysis

Averaged model does not capture ripple dynamics

## 9. Conclusion

This live script demonstrates a structured workflow for modeling, simulating, and validating a Buck converter in Simulink.

The averaged model enables efficient control design, while the switched model provides accurate validation of converter dynamics.

## 10. References

[1] R. Erickson and D. Maksimovic, Fundamentals of Power Electronics

[2] MathWorks Documentation: DC-DC Converter Modeling